VFC INITIAL PA-44 CHECK RIDE EXAM- JANUARY 2021

PILOT_____________________						DATE____________________

INSTRUCTOR_______________					              SCORE:_____________/62  /=_______%

- The PA-44-180 Seminole total + useable fuel capacities are __110____ USG and __108____ USG respectively. For training purposes, the A/C is normally fueled to __80____ USG total. [POH 2-6, 1st flight training]. (3)

- Using average power settings of 65%, what is the approximate endurance of the PA44 with fuel at 80USG?___3:50_____ [POH 5-25, 5-26]    (1)

- During pre-flight for a local recreational flight, the minimum oil level per engine that should be read on the dipstick is __4_____ QT, and the maximum should be __4.5____QT. [1st flight training].   (2)

- During pre-flight, you confirm that the landing gear selector switch is in the down position, then turn ON the master switch, and notice the 3 green “gear down” lights do not appear illuminated. What is the most likely cause? ___The NAV lights switch is on____ [POH 7-9].      (1)

- List the following operational speeds in KIAS:	[POH 2-1, 2-2]    (9)

VA at 2700lbs	 _112________

VA at 3800 lbs              _135________

Vmc	                 _56________

VLO		 _109________

Vs		 _57________

Vyse		 _88________

VFE		 _111________

VLE		 __140_______

VNE		 _202________

- What is the normal operating range of the engines fuel pressure? _0.5 to 8.0 psi___ [POH 2-4]    (1)

- In flight, if there is a failure or leak in the landing gear hydraulic system, what will occur?___Landing gear will extend to down position___ [POH 7-7, 4-22].   (1)

- At the beginning and end of the “Post Start” checklist, the fuel selectors are switched from On to X-FEED alternately and returned to ON at the beginning of the run up. Why is this done? ___To confirm proper functioning of the cross-feed system_  [Normal checklist].    (1)

- During pre-start, when deciding how much to prime the engines, what is the best method to determine “cold” start versus “warm” start? __Check engine oil temperature____. If cold, the engines should be primed __9____ times, and if warm it should be primed _4 to 5_ times. [1st flight training].    (3)

- During the run up checklist, there are 3 levels of propeller governor checks; the feather check, the exercise, and the governing check. During the governing check, both prop levels should be reduced to _1800__ RPM, and each throttle should be advanced individually_ to indicate a _slight_ increase in manifold pressure. A functioning governor will be indicated by __no_ change in engine RPM.  [POH 4-18, 1st flight training]    (4)

- If during the run up magneto checks, an engine RPM drop in excess of 175 RPM or excessive engine roughness is observed, the pilot should leave the power at _2000_RPM and _leaned to peak EGT__for at least _10____ seconds. [POH 4-18]     (3)

- The PA44 takeoff safety briefing should state that VR is _75__KTS, Blue Line VYSE is __88_KTS, and Normal Climb is __105__KTS. [POH 4-20, 1st flight training]    (3)

- Technically, to produce best power performance in cruise, the engine mixtures should be leaned to _125_____ degrees Fahrenheit rich of _peak EGT__. [POH 5-26, flight training].    (2)

- To enter a climb from cruise, the correct sequence to set the climb power is enrichen the mixtures, set props_to_2500_first, and then set the __throttles_to 25” Hg__. [SOP’s, 1st flight training].  (2)

- What is the purpose of the engine cowl flaps? _to regulate engine cooling / (control CHT and oil temp)_. They should be open for _climbs_, and usually closed during _cruise and descent___. [POH 4-21].   (3)

- Cylinder head temperature should be maintained below _435_ degrees Fahrenheit in climb or high performance cruise and below __400__ degrees Fahrenheit during economy cruise. [POH 2-5, 4-21].    (2)

- Oil temperatures should be maintained between _75___ and _245_degrees Fahrenheit with use of the power controls and cowl flaps. [POH 2-4]       (1)

- Explain the landing gear manual extension procedure. [POH 3-7, VFC PA44 Emergency Checklist]  (4)

1)__Use checklist. Slow to 100 KIAS____________________________

2)__Gear selector -- Down___________________________________

3)_  Emergency Gear extension knob_-- Pull_____________________

4)__Check Gear lights  -- 3 Green,_leave knob extended___________

- What makes the gear go down when you use the manual extension system?__It releases the hydraulic pressure. Gravity and a (over center) spring pull the gear down__. [POH 7-7].    (1)

- In case of landing gear loss of hydraulic fluid, are the brakes still effective? ___Yes. Separate hydraulic system/resevoir___. [POH 7-12].   (1)

- What 3 conditions will cause the gear warning horn to activate? [POH 7-9]     (3)

1)_Gear not down_and locked with MP below 14” Hg on either engine.

2)_Gear selector in UP position with aircraft on ground.____________

3)_Gear selector in the UP position_and wing flaps extended to beyond 10 degrees (25 or 40 degrees position).

- Under all circumstances, what are the 3 most important and vital actions in the event of an engine power loss?

1)_Control  --  Rudder opposite yaw._____________

2)_Power  --  Full Mixture, Pitch, Power.__________

3)_Drag  --  Gear up, Flaps up.__________________      (3)

- What are the voltage and capacity of the battery and alternators respectively?     (2)

Battery:  _12 volt, 35 amps._____________

Alternators: _14 volts, 60 amps._________

- What is the first main indication of an alternator failure and how can it be immediately confirmed?    (2)

ALT annunciator ON_ and _L or R ammeter reading Zero.___

- Using the attached form….                                                                          (4)

Based on the fuel flow from Question #2, calculate and plot a FHDP weight and balance for:

-A summer standard weight male pilot + front passenger

-A summer standard female +10 year old child as a rear seat passenger

-A 25lb picnic basket in the cargo area

-Take off with max possible fuel

-Landing after a 1 ½ hour cruise trip.